TECHNICAL RESPONSE – 1.5KVA UPS

Submitted by:
Dolphin Professional Services Limited (DPSL)
Prepared by: Erick Matondo – Systems Engineer

1. Introduction

We acknowledge receipt of the RFQ and hereby submit our technical response for the supply, delivery, installation, and support of 1.5KVA Uninterruptible Power Supply (UPS) units.
As indicated, this is a repeat request matching the 10 units previously supplied; therefore, we have aligned all technical specifications to the same standard to ensure full compatibility and continuity across your environment.

2. Proposed UPS Model

We are proposing the same 1.5KVA UPS model delivered in the previous procurement, ensuring operational uniformity, ease of maintenance, and consistent performance across all deployed units.

3. Compliance With RFQ Requirements

Below is the technical compliance statement based on standard 1.5KVA line-interactive or double-conversion UPS units normally deployed in enterprise environments.

A. Electrical Specifications

| Specification | Proposed Response |
| --- | --- |
| Capacity | 1.5KVA / 900W |
| Topology | Line-interactive / Online double-conversion (match previous sourced model) |
| Input Voltage Range | Wide AVR range to handle unstable mains (typically 145–280V) |
| Output Voltage | Pure sinewave 230V AC |
| Input Frequency | 50/60Hz auto-sensing |
| Efficiency | ≥ 90% |

B. Battery System

| Feature | Description |
| --- | --- |
| Type | Sealed, maintenance-free VRLA |
| Runtime | 10–20 minutes at 50% load (depending on model) |
| Recharge Time | 90% capacity within 4–6 hours |
| Hot-swap Support | Available if applicable to previous model |

C. Protection Features

- Automatic Voltage Regulation (AVR)

- Surge protection and spike suppression

- Overload and short-circuit protection

- Intelligent thermal management

- Battery overcharge/over-discharge protection

D. Management & Interface

- LCD display for input/output status, battery health, and load level

- USB/Serial communication ports

- UPS management software compatibility

- Audible alarms for battery mode, low battery, overload, and fault conditions

4. Environmental Requirements

- Operating temperature: 0°C – 40°C

- Humidity: 0% – 95% (non-condensing)

- Noise level: < 45dB

- Suitable for office/server room environments

5. Warranty & Support

DPSL will provide:

- 12–24 months manufacturer warranty (depending on previous model agreement)

- Local support and service through DPSL technical teams

- Replacement/repair coverage per SLA

6. Delivery & Installation

- Delivery within the timeline specified in the RFQ.

- On-site installation and commissioning included.

- Post-installation testing and handover documentation will be provided.

7. Conclusion

Our proposed solution fully aligns with the specifications requested and maintains consistency with the previous batch of 10 UPS units supplied to the Postal Corporation of Kenya. We remain committed to delivering reliable power protection solutions and providing continued technical support.